Question to Gemini

Hi, I have 7 Cisco routers with the following outputs:

CORE-PE#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         unassigned      YES NVRAM  up                    up

GigabitEthernet0/0.10      192.168.255.1   YES manual up                    up

GigabitEthernet0/0.100     8.8.8.254       YES manual up                    up

GigabitEthernet0/1         10.252.0.1      YES manual up                    up

GigabitEthernet0/2         10.252.0.5      YES manual up                    up

GigabitEthernet0/3         unassigned      YES NVRAM  administratively down down

Loopback0                  10.10.10.10     YES manual up                    up

CORE-PE#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

GigabitEthernet0/0.10 is up, line protocol is up

Internet address is 192.168.255.1/30

GigabitEthernet0/0.100 is up, line protocol is up

Internet address is 8.8.8.254/24

GigabitEthernet0/1 is up, line protocol is up

Internet address is 10.252.0.1/30

GigabitEthernet0/2 is up, line protocol is up

Internet address is 10.252.0.5/30

GigabitEthernet0/3 is administratively down, line protocol is down

Loopback0 is up, line protocol is up

Internet address is 10.10.10.10/32

CORE-PE#

PE-01#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         80.200.200.5    YES manual up                    up

GigabitEthernet0/1         10.252.0.2      YES manual up                    up

GigabitEthernet0/2         80.200.200.1    YES manual up                    up

GigabitEthernet0/3         unassigned      YES NVRAM  administratively down down

Loopback0                  20.20.20.20     YES manual up                    up

PE-01#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

Internet address is 80.200.200.5/30

GigabitEthernet0/1 is up, line protocol is up

Internet address is 10.252.0.2/30

GigabitEthernet0/2 is up, line protocol is up

Internet address is 80.200.200.1/30

GigabitEthernet0/3 is administratively down, line protocol is down

Loopback0 is up, line protocol is up

Internet address is 20.20.20.20/32

PE-01#

PE-02#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         80.200.200.9    YES manual up                    up

GigabitEthernet0/1         80.200.200.13   YES manual up                    up

GigabitEthernet0/2         10.252.0.6      YES manual up                    up

GigabitEthernet0/3         unassigned      YES NVRAM  administratively down down

Loopback0                  30.30.30.30     YES manual up                    up

PE-02#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

Internet address is 80.200.200.9/30

GigabitEthernet0/1 is up, line protocol is up

Internet address is 80.200.200.13/30

GigabitEthernet0/2 is up, line protocol is up

Internet address is 10.252.0.6/30

GigabitEthernet0/3 is administratively down, line protocol is down

Loopback0 is up, line protocol is up

Internet address is 30.30.30.30/32

PE-02#

CPE-01#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         80.200.200.6    YES manual up                    up

GigabitEthernet0/1         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/2         172.16.32.5     YES manual up                    up

GigabitEthernet0/3         172.16.33.5     YES manual up                    up

Loopback0                  1.1.1.1         YES manual up                    up

CPE-01#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

Internet address is 80.200.200.6/30

GigabitEthernet0/1 is administratively down, line protocol is down

GigabitEthernet0/2 is up, line protocol is up

Internet address is 172.16.32.5/29

GigabitEthernet0/3 is up, line protocol is up

Internet address is 172.16.33.5/29

Loopback0 is up, line protocol is up

Internet address is 1.1.1.1/32

CPE-01#

CPE-02#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         80.200.200.10   YES manual up                    up

GigabitEthernet0/1         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/2         172.16.33.6     YES manual up                    up

GigabitEthernet0/3         172.16.32.6     YES manual up                    up

Loopback0                  2.2.2.2         YES manual up                    up

CPE-02#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

Internet address is 80.200.200.10/30

GigabitEthernet0/1 is administratively down, line protocol is down

GigabitEthernet0/2 is up, line protocol is up

Internet address is 172.16.33.6/29

GigabitEthernet0/3 is up, line protocol is up

Internet address is 172.16.32.6/29

Loopback0 is up, line protocol is up

Internet address is 2.2.2.2/32

CPE-02#

CPE-04#show ip interface brief

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         192.168.15.254  YES manual up                    up

GigabitEthernet0/1         80.200.200.14   YES manual up                    up

GigabitEthernet0/2         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/3         unassigned      YES NVRAM  administratively down down

Loopback0                  4.4.4.4         YES manual up                    up

CPE-04#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

Internet address is 192.168.15.254/23

GigabitEthernet0/1 is up, line protocol is up

Internet address is 80.200.200.14/30

GigabitEthernet0/2 is administratively down, line protocol is down

GigabitEthernet0/3 is administratively down, line protocol is down

Loopback0 is up, line protocol is up

Internet address is 4.4.4.4/32

CPE-04#

CPE-05#show ip interface br

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         172.32.23.254   YES manual up                    up

GigabitEthernet0/1         unassigned      YES NVRAM  administratively down down

GigabitEthernet0/2         80.200.200.2    YES manual up                    up

GigabitEthernet0/3         unassigned      YES NVRAM  administratively down down

Loopback0                  5.5.5.5         YES manual up                    up

CPE-05#show protocols

Global values:

Internet Protocol routing is enabled

GigabitEthernet0/0 is up, line protocol is up

Internet address is 172.32.23.254/24

GigabitEthernet0/1 is administratively down, line protocol is down

GigabitEthernet0/2 is up, line protocol is up

Internet address is 80.200.200.2/30

GigabitEthernet0/3 is administratively down, line protocol is down

Loopback0 is up, line protocol is up

Internet address is 5.5.5.5/32

CPE-05#

CORE-PE, PE-01 and PE-02 are AS 8654.

CPE-01 and CPE-02 are AS 65001.

CPE-05 is AS 65002.

CPE-04 is AS 65003.

And the tasks definition is the following:

"BGP

The following tasks must be implemented for the BGP configuration:

- AS 8654 must advertise only a default route (0.0.0.0/0) to the customer CPE. Ensure that no other prefixes are advertised.

- The router CORE-PE will act as a Route Reflector for internal BGP peers within AS 8654.

- Authentication must be enabled on all eBGP peerings to ensure secure session establishment."

How can we configure it?

Of course. Based on your requirements and the provided interface information, here is the BGP configuration for all seven routers.

The configuration establishes an iBGP full mesh within AS 8654 using CORE-PE as a Route Reflector, sets up authenticated eBGP peerings with the customer routers, and ensures that only a default route is advertised from the core to the customers.

### Network Topology

First, let's visualize the connections based on the shared subnets from your output.

- iBGP (AS 8654): CORE-PE <--> PE-01 & PE-02 (using loopbacks)

- eBGP (AS 8654 <--> Customers):

  - PE-01 <--> CPE-01 (AS 65001)

  - PE-01 <--> CPE-05 (AS 65002)

  - PE-02 <--> CPE-02 (AS 65001)

  - PE-02 <--> CPE-04 (AS 65003)

- iBGP (AS 65001): CPE-01 <--> CPE-02 (using loopbacks)

## Provider Routers (AS 8654) Configuration

A crucial prerequisite for iBGP to function correctly using loopback interfaces is to have an Interior Gateway Protocol (IGP), like OSPF or EIGRP, running within AS 8654. This ensures that the loopback addresses of all internal routers (10.10.10.10, 20.20.20.20, 30.30.30.30) are reachable from one another.

### CORE-PE (AS 8654) - Route Reflector

This router will act as the central point for iBGP updates, reflecting routes to its clients (PE-01, PE-02).

Cisco CLI

ip route 20.20.20.20 255.255.255.255 10.252.0.2

!

router bgp 8654

bgp router-id 10.10.10.10

bgp log-neighbor-changes

!

! Peer with PE-01 (iBGP RR Client)

neighbor 20.20.20.20 remote-as 8654

neighbor 20.20.20.20 update-source Loopback0

neighbor 20.20.20.20 route-reflector-client

neighbor 20.20.20.20 next-hop-self

!

! Peer with PE-02 (iBGP RR Client)

neighbor 30.30.30.30 remote-as 8654

neighbor 30.30.30.30 update-source Loopback0

neighbor 30.30.30.30 route-reflector-client

neighbor 30.30.30.30 next-hop-self

!

### PE-01 (AS 8654) - RR Client

This router peers internally with the Route Reflector and externally with CPE-01 and CPE-05. It originates the default route to these customers.

Cisco CLI

!

router bgp 8654

bgp router-id 20.20.20.20

bgp log-neighbor-changes

!

! Peer with the Route Reflector (CORE-PE)

neighbor 10.10.10.10 remote-as 8654

neighbor 10.10.10.10 update-source Loopback0

!

! Peer with CPE-01 (Customer AS 65001)

neighbor 80.200.200.6 remote-as 65001

neighbor 80.200.200.6 password YourSecretPassword

neighbor 80.200.200.6 route-map ONLY-DEFAULT out

!

! Peer with CPE-05 (Customer AS 65002)

neighbor 80.200.200.2 remote-as 65002

neighbor 80.200.200.2 password YourSecretPassword

neighbor 80.200.200.2 route-map ONLY-DEFAULT out

!

!

! Create a prefix-list to match only the default route

ip prefix-list DEFAULT-ROUTE seq 5 permit 0.0.0.0/0

!

! Create a route-map to filter outgoing advertisements

route-map ONLY-DEFAULT permit 10

match ip address prefix-list DEFAULT-ROUTE

!

Note: The command default-originate could also be used, but a route-map provides more granular control and explicitly ensures only the default route is sent.

### PE-02 (AS 8654) - RR Client

This router is configured similarly to PE-01, peering with the Route Reflector and its respective customer routers, CPE-02 and CPE-04.

Cisco CLI

!

router bgp 8654

bgp router-id 30.30.30.30

bgp log-neighbor-changes

!

! Peer with the Route Reflector (CORE-PE)

neighbor 10.10.10.10 remote-as 8654

neighbor 10.10.10.10 update-source Loopback0

!

! Peer with CPE-02 (Customer AS 65001)

neighbor 80.200.200.10 remote-as 65001

neighbor 80.200.200.10 password YourSecretPassword

neighbor 80.200.200.10 route-map ONLY-DEFAULT out

!

! Peer with CPE-04 (Customer AS 65003)

neighbor 80.200.200.14 remote-as 65003

neighbor 80.200.200.14 password YourSecretPassword

neighbor 80.200.200.14 route-map ONLY-DEFAULT out

!

!

! Create a prefix-list to match only the default route

ip prefix-list DEFAULT-ROUTE seq 5 permit 0.0.0.0/0

!

! Create a route-map to filter outgoing advertisements

route-map ONLY-DEFAULT permit 10

match ip address prefix-list DEFAULT-ROUTE

!

## Customer Routers (CPE) Configuration

The customer routers simply need to establish an authenticated eBGP session with their corresponding provider edge (PE) router. Routers in the same customer AS (CPE-01 and CPE-02) should also peer with each other via iBGP.

### CPE-01 (AS 65001)

Cisco CLI

!

router bgp 65001

bgp router-id 1.1.1.1

bgp log-neighbor-changes

!

! Peer with Provider (PE-01)

neighbor 80.200.200.5 remote-as 8654

neighbor 80.200.200.5 password YourSecretPassword

!

! iBGP Peer with CPE-02

neighbor 2.2.2.2 remote-as 65001

neighbor 2.2.2.2 update-source Loopback0

!

### CPE-02 (AS 65001)

Cisco CLI

!

router bgp 65001

bgp router-id 2.2.2.2

bgp log-neighbor-changes

!

! Peer with Provider (PE-02)

neighbor 80.200.200.9 remote-as 8654

neighbor 80.200.200.9 password YourSecretPassword

!

! iBGP Peer with CPE-01

neighbor 1.1.1.1 remote-as 65001

neighbor 1.1.1.1 update-source Loopback0

!

### CPE-04 (AS 65003)

Cisco CLI

!

router bgp 65003

bgp router-id 4.4.4.4

bgp log-neighbor-changes

!

! Peer with Provider (PE-02)

neighbor 80.200.200.13 remote-as 8654

neighbor 80.200.200.13 password YourSecretPassword

!

### CPE-05 (AS 65002)

Cisco CLI

!

router bgp 65002

bgp router-id 5.5.5.5

bgp log-neighbor-changes

!

! Peer with Provider (PE-01)

neighbor 80.200.200.1 remote-as 8654

neighbor 80.200.200.1 password YourSecretPassword

!

## Verification Commands

After applying the configurations, you can use these commands to verify that everything is working as expected. ✅

- Check BGP session status: Run this on all routers. You should see the state as Established.

Shell

show ip bgp summary

- Verify Route Reflector clients: On CORE-PE, this command will show which neighbors are RR clients.

Shell

show ip bgp neighbors

- Confirm default route advertisement: On a PE router (e.g., PE-01), check what you are sending to a customer (e.g., CPE-01). You should only see the 0.0.0.0/0 prefix.

Shell

show ip bgp neighbors 80.200.200.6 advertised-routes

- Confirm default route reception: On a CPE router (e.g., CPE-01), check the routes received from the provider.

Shell

show ip bgp neighbors 80.200.200.5 received-routes