CORE-PE	2

PE-01	4

PE-02	6

CPE-01	7

CPE-02	9

CPE-04	11

CPE-05	13

CLOUDSW	15

CLOUDFW	17

DSW1	19

DSW2	21

ASW1	23

ASW2	25

# CORE-PE

enable

configure terminal

hostname CORE-PE

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 10.10.10.10 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.255.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/0.100

encapsulation dot1Q 100

ip address 8.8.8.254 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.252.0.1 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 10.252.0.5 255.255.255.252

no shutdown

exit

copy running-config startup-config

# PE-01

enable

configure terminal

hostname PE-01

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 20.20.20.20 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0

ip address 80.200.200.5 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.252.0.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 80.200.200.1 255.255.255.252

no shutdown

exit

copy running-config startup-config

# PE-02

enable

configure terminal

hostname PE-02

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 30.30.30.30 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0

ip address 80.200.200.9 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1

ip address 80.200.200.13 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 10.252.0.6 255.255.255.252

no shutdown

exit

copy running-config startup-config

# CPE-01

enable

configure terminal

hostname CPE-01

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 1.1.1.1 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0

ip address 80.200.200.6 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 172.16.32.5 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/3

ip address 172.16.33.5 255.255.255.248

no shutdown

exit

copy running-config startup-config

# CPE-02

enable

configure terminal

hostname CPE-02

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 2.2.2.2 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0

ip address 80.200.200.10 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/2

ip address 172.16.33.6 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/3

ip address 172.16.32.6 255.255.255.248

no shutdown

exit

copy running-config startup-config

# CPE-04

enable

configure terminal

hostname CPE-04

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 4.4.4.4 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0

ip address 192.168.15.254 255.255.254.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 80.200.200.14 255.255.255.252

no shutdown

exit

copy running-config startup-config

# CPE-05

enable

configure terminal

hostname CPE-05

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

enable

configure terminal

interface loopback 0

ip address 5.5.5.5 255.255.255.255

no shutdown

exit

interface gigabitEthernet 0/0

ip address 172.32.23.254 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/2

ip address 80.200.200.2 255.255.255.252

no shutdown

exit

copy running-config startup-config

# CLOUDSW

enable

configure terminal

hostname CLOUDSW

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

configure terminal

vlan 10

name herningco

exit

vlan 100

name Cloud

exit

interface gigabitEthernet 0/0

switchport trunk encapsulation dot1q

switchport mode trunk

no shutdown

exit

interface gigabitEthernet 0/1

switchport trunk encapsulation dot1q

switchport mode trunk

no shutdown

exit

interface gigabitEthernet 0/2

switchport mode access

no shutdown

exit

copy running-config startup-config

# CLOUDFW

Password for exec mode (enable) is Cisco1@3

enable

Cisco1@3

configure terminal

hostname CLOUDFW

interface gigabitEthernet 0/0

ip address 192.168.50.254 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1.10

vlan 10

nameif herningco

ip address 192.168.255.2 255.255.255.252

no shutdown

exit

interface gigabitEthernet 0/1.100

vlan 100

nameif Cloud

ip address 8.8.8.10 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

no nameif

no security-level

no ip address

no shutdown

exit

enable

copy running-config startup-config

# DSW1

enable

configure terminal

hostname DSW1

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

configure terminal

vlan 100

name SERVER

exit

vlan 10

name CLIENT

exit

vlan 999

name UnusedPorts

exit

vlan 99

name Native

exit

vlan 200

name CPEDS1

exit

configure terminal

interface gigabitEthernet 0/0

switchport mode access

switchport access vlan 200

no shutdown

exit

interface gigabitEthernet 0/1

switchport mode access

switchport access vlan 200

no shutdown

exit

interface gigabitEthernet 0/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface gigabitEthernet 0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface vlan 200

ip address 172.16.32.1 255.255.255.248

no shutdown

exit

# DSW2

enable

configure terminal

hostname DSW2

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

configure terminal

vlan 100

name SERVER

exit

vlan 10

name CLIENT

exit

vlan 999

name UnusedPorts

exit

vlan 99

name Native

exit

vlan 201

name CPEDS2

exit

configure terminal

interface gigabitEthernet 0/0

switchport mode access

switchport access vlan 201

no shutdown

exit

interface gigabitEthernet 0/1

switchport mode access

switchport access vlan 201

no shutdown

exit

interface gigabitEthernet 0/2

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface gigabitEthernet 0/3

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface vlan 201

ip address 172.16.33.1 255.255.255.248

no shutdown

exit

# ASW1

enable

configure terminal

hostname ASW1

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

configure terminal

vlan 100

name SERVER

exit

vlan 10

name CLIENT

exit

vlan 999

name UnusedPorts

exit

vlan 99

name Native

exit

configure terminal

interface gigabitEthernet 0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface gigabitEthernet 0/1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface gigabitEthernet 0/2

switchport mode access

switchport access vlan 100

no shutdown

exit

interface gigabitEthernet 0/3

switchport mode access

switchport access vlan 999

no shutdown

exit

# ASW2

enable

configure terminal

hostname ASW2

enable

configure terminal

ip domain name isp.denmark

crypto key generate rsa modulus 2048

username Administrator secret Passw0rd!

line vty 0 4

transport input ssh

login local

exit

ip ssh version 2

exit

configure terminal

vlan 100

name SERVER

exit

vlan 10

name CLIENT

exit

vlan 999

name UnusedPorts

exit

vlan 99

name Native

exit

configure terminal

interface gigabitEthernet 0/0

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface gigabitEthernet 0/1

switchport trunk encapsulation dot1q

switchport mode trunk

switchport trunk native vlan 99

switchport trunk allowed vlan 10,99,100

switchport trunk allowed vlan remove 999

no shutdown

exit

interface gigabitEthernet 0/2

switchport mode access

switchport access vlan 10

no shutdown

exit

interface gigabitEthernet 0/3

switchport mode access

switchport access vlan 999

no shutdown

exit