To configure DMVPN with DHCP snooping using your parameters, you'll set up the hub and spokes for DMVPN Phase 3, enable NHRP authentication, and apply a specific IPsec encryption profile. The DHCP snooping configuration at the spoke sites remains the same.

The key changes for Phase 3 are the ip nhrp redirect command on the hub and the ip nhrp shortcut command on the spoke tunnel interfaces. This allows spokes to communicate directly with each other after initially contacting the hub, creating a more efficient spoke-to-spoke traffic flow.

1. IPsec (Crypto) Configuration

First, define the encryption parameters which will be applied to the tunnel interfaces on both the hub and spokes. This configuration must match on all routers.

IPsec Phase 1 (ISAKMP Policy):

! Defines how the routers establish a secure management connection

crypto isakmp policy 10

encr aes 256

hash sha256

authentication pre-share

group 14

! Set the pre-shared key for the DMVPN peer (hub's public IP for spokes, or an address range)

crypto isakmp key <your_secure_preshared_key> address 0.0.0.0 0.0.0.0

IPsec Phase 2 (Transform Set & Profile):

! Defines how the actual data is encrypted

crypto ipsec transform-set TSET esp-aes 256 esp-sha256-hmac

mode tunnel

! Combine Phase 1 and Phase 2 settings into a profile

crypto ipsec profile DMVPN_PROFILE

set transform-set TSET

2. DMVPN Hub Router Configuration

The hub is configured to act as the NHRP server and redirect spoke-to-spoke communication.

! Interface facing the internet

interface GigabitEthernet0/0

ip address <hub_public_ip_address> <subnet_mask>

no shutdown

! DMVPN Tunnel Interface

interface Tunnel100

ip address 192.168.100.1 255.255.255.0

no ip redirects

! Enable NHRP authentication with your chosen password

ip nhrp authentication <your_nhrp_password>

ip nhrp map multicast dynamic

! Set NHRP Network ID

ip nhrp network-id 100

! HUB ONLY: Enables Phase 3 spoke-to-spoke shortcut traffic

ip nhrp redirect

tunnel source GigabitEthernet0/0

tunnel mode gre multipoint

! Apply the IPsec profile created earlier

tunnel protection ipsec profile DMVPN_PROFILE

3. DMVPN Spoke Router Configuration

Spokes are configured to register with the hub and to initiate direct tunnels to other spokes when needed.

! Interface facing the internet

interface GigabitEthernet0/0

ip address dhcp ! or a static public IP

no shutdown

! DMVPN Tunnel Interface

interface Tunnel100

ip address 192.168.100.X 255.255.255.0 ! Unique IP for each spoke

no ip redirects

! NHRP authentication password must match the hub

ip nhrp authentication <your_nhrp_password>

ip nhrp map 192.168.100.1 <hub_public_ip>

ip nhrp map multicast <hub_public_ip>

! Set NHRP Network ID to match the hub

ip nhrp network-id 100

! Define the Next Hop Server (the hub)

ip nhrp nhs 192.168.100.1

! SPOKE ONLY: Enables Phase 3 spoke-to-spoke shortcut traffic

ip nhrp shortcut

tunnel source GigabitEthernet0/0

tunnel mode gre multipoint

! Apply the same IPsec profile

tunnel protection ipsec profile DMVPN_PROFILE

4. DHCP Relay and Snooping Configuration

This part of the configuration remains unchanged from the previous setup.

* DHCP Server: A central DHCP server is configured at the hub site.

* Hub as Relay: The hub's Tunnel100 interface is configured with ip helper-address <dhcp_server_ip> to forward DHCP requests from spokes.

* DHCP Snooping on Spoke Switch:

* ip dhcp snooping is enabled globally.

* ip dhcp snooping vlan <vlan_id> is enabled for the client VLAN.

* The switch port connected to the spoke router's LAN interface is set as trusted using ip dhcp snooping trust. All other client-facing ports remain untrusted, blocking rogue DHCP servers.

This setup ensures a secure, scalable, and efficient DMVPN Phase 3 network with centralized IP address management and local LAN security.

OTRAS RESPUESTAS (CHATGPT):

DAI:

La Dynamic ARP Inspection (DAI) es una característica de seguridad en switches Cisco que protege contra ataques de spoofing ARP, como el Man-in-the-Middle (MITM). DAI valida los mensajes ARP en una red para asegurarse de que las IP se corresponden con las direcciones MAC correctas.

## ¿Cómo funciona DAI?

DAI intercepta todas las solicitudes y respuestas ARP en interfaces no confiables y compara las direcciones IP-MAC contra una base de datos de bindings seguros conocida como DHCP snooping binding table. Si no hay coincidencia, el paquete se descarta.

## Requisitos previos

- DHCP Snooping debe estar habilitado para que DAI funcione correctamente.

- Las interfaces que conectan a otros switches o dispositivos de red deben marcarse como confiables (trusted).

- Las interfaces de acceso a clientes deben ser no confiables (untrusted), que es el comportamiento por defecto.

## Pasos para implementar DAI en un switch Cisco

### 1️⃣ Habilitar DHCP Snooping

ip dhcp snooping

ip dhcp snooping vlan 10

### 2️⃣ Habilitar DAI para las VLAN necesarias

ip arp inspection vlan 10

### 3️⃣ Configurar interfaces confiables (uplinks, conexiones a routers)

interface GigabitEthernet1/0/1

description Uplink to Distribution Switch

ip arp inspection trust

### 4️⃣ Verificar la tabla DHCP snooping

show ip dhcp snooping binding

Esta tabla es usada por DAI para validar los paquetes ARP.

## 🧪 Comprobación de funcionamiento

Puedes verificar el estado de DAI con:

show ip arp inspection

show ip arp inspection statistics

Y por VLAN específica:

show ip arp inspection vlan 10

## 🔄 Validación sin DHCP (binding estático)

Si usas direcciones IP estáticas, puedes añadir manualmente un binding:

ip source binding 0011.2233.4455 vlan 10 192.168.10.10 interface GigabitEthernet1/0/2

Esto permite que DAI valide esa dirección sin DHCP snooping.

Pero se pueden fijar manualmente las ip's y mac's?

###### Asistente de Redes Cisco dijo:

Sí, puedes fijar (configurar manualmente) las relaciones entre IP, MAC, VLAN e interfaz para que se almacenen como entradas estáticas en la tabla de DHCP Snooping. Estas entradas también son utilizadas por funciones como Dynamic ARP Inspection (DAI) y IP Source Guard para validar tráfico de clientes que usan IP estáticas (sin pasar por DHCP).

## 🧱 Comando para fijar un binding manualmente

ip source binding <MAC-ADDRESS> vlan <VLAN-ID> <IP-ADDRESS> interface <INTERFACE>

### 🧾 Ejemplo:

ip source binding 0011.2233.4455 vlan 10 192.168.10.50 interface GigabitEthernet1/0/10

Esto crea un binding estático que asocia:

- MAC: 00:11:22:33:44:55

- IP: 192.168.10.50

- VLAN: 10

- Interfaz: G1/0/10

## 🔍 Verificación:

Usa el siguiente comando para ver todas las entradas, tanto dinámicas como estáticas:

show ip dhcp snooping binding

Las entradas manuales se marcarán como static.

DMVPN (CHATGPT)

La DMVPN Fase 3 es una evolución de las fases anteriores y permite una conmutación directa entre spokes con actualización dinámica de rutas, optimizando el tráfico (evitando el hub después del primer contacto).

### 🧠 Características clave de DMVPN Fase 3:

- Soporta comunicación spoke-to-spoke directa.

- El hub actúa inicialmente como router de reenvío, pero luego los spokes actualizan sus rutas para comunicarse directamente.

- Usa NHRP redirect y NHRP shortcut.

## 🔧 Topología y direcciones IP (idéntico a Fase 2)

- Hub: 10.0.0.1

- Spoke1: 10.0.0.2

- Spoke2: 10.0.0.3

## 1️⃣ Configuración del HUB (Fase 3)

interface Tunnel0

ip address 10.0.0.1 255.255.255.0

no ip redirects

ip nhrp authentication dmvpnkey

ip nhrp map multicast dynamic

ip nhrp network-id 1

ip nhrp redirect

tunnel source GigabitEthernet0/0

tunnel mode gre multipoint

tunnel key 100

tunnel protection ipsec profile DMVPN-PROFILE

## 2️⃣ Configuración de los Spokes

interface Tunnel0

ip address 10.0.0.X 255.255.255.0   ! X = 2, 3, etc.

no ip redirects

ip nhrp authentication dmvpnkey

ip nhrp map 10.0.0.1 200.1.1.1

ip nhrp map multicast 200.1.1.1

ip nhrp network-id 1

ip nhrp nhs 10.0.0.1

ip nhrp shortcut

tunnel source GigabitEthernet0/0

tunnel mode gre multipoint

tunnel key 100

tunnel protection ipsec profile DMVPN-PROFILE

## 3️⃣ Configuración de IPsec (idéntico a Fase 2)

(igual en hub y spokes)

crypto isakmp policy 10

encr aes 256

hash sha

authentication pre-share

group 2

crypto isakmp key cisco123 address 0.0.0.0

crypto ipsec transform-set DMVPN-SET esp-aes 256 esp-sha-hmac

crypto ipsec profile DMVPN-PROFILE

set transform-set DMVPN-SET

## ✅ Verificación (comandos clave)

En el hub:

show dmvpn

show ip nhrp

En los spokes:

show dmvpn

show ip nhrp

traceroute <otro spoke>

El traceroute debe mostrar que al principio va al HUB y luego, con el shortcut, va directo al otro Spoke.